## Standard Portable Container Dimensions & Specifications

Understanding standard ISO container sizes helps you choose the right unit for storage, shipping, or specialized projects. Most containers feature a uniform width of 8 feet, with varying lengths and heights.

| Container Type | Exterior (L x W x H) | Interior (L x W x H) | Capacity (Cu. Ft.) | Max Payload |
| --- | --- | --- | --- | --- |
| 20' Standard | 19' 10.5" x 8' x 8' 6" | 19' 4" x 7' 8" x 7' 10" | ~1,170 | ~62,000 lbs |
| 40' Standard | 40' x 8' x 8' 6" | 39' 5" x 7' 8" x 7' 10" | ~2,390 | ~58,000 lbs |
| 40' High Cube | 40' x 8' x 9' 6" | 39' 5" x 7' 8" x 8' 10" | ~2,690 | ~58,000 lbs |
| 10' Standard | 9' 9.75" x 8' x 8' 6" | 9' 3" x 7' 8" x 7' 10" | ~560 | ~20,000 lbs |

### Key Features and Considerations

- Structural Integrity: Built from corrosion-resistant Corten steel to meet [ISO 668 standards](https://www.iso.org/standard/76912.html) for stacking and intermodal transport.

- Door Openings: Standard doors are typically 7' 8" wide. Standard height units offer 7' 5" of clearance, while High Cubes provide 8' 5".

- Tare Weight: An empty 20' container weighs roughly 4,500 lbs, while a 40' unit is about 8,300 lbs.

- Space Comparison: A 20-foot container typically holds the contents of a 1-2 bedroom apartment (approx. 10 standard pallets), while a 40-foot container fits a 3-4 bedroom home.

### Specialized Sizes

For domestic North American transport, 53-foot High Cube containers are common, offering extra width (8' 6") and height (9' 6") for increased volume in over-the-road and rail service.